Indicator Fact Sheet – Theme WASTE

CSI 024 Urban Waste Water Treatment

Key message  Wastewater treatment in Croatia has been improved over last few years. Public sewerage collects and conducts household wastewater along with treated or untreated industrial wastewater. Precipitation that washes and carries contamination from roads and nearby agricultural and other land, is also connected to public sewerage system.

Figure 1: Discharged water from public sewerage system

Source: Croatian Bureau of Statistics (CBS)

Prepared by: Croatian Environmental Agency (CEA)

Figure 2: Sewerage wastewaters treatment

Source: CBS; Prepared by: CEA

Results and assessment

Policy relevance

The environment protection from adverse effects of urban wastewaters discharges has been regulated by several acts and regulations e.g. Water Act, Public Utility Management Act etc. There is also legal obligation to collect data on emissions and for wastewaters. Those data are processed by the Croatian Environment Agency.

Assessment

The sewerage network construction trend is on increase as well as the connection to the public sewerage network. In 2004, total length of city and main sewers was 5.996 km and 1.314 km, respectively. Croatian average of connection to the public sewerage system is higher than 40% (appr. 70% in large cities and under 40% in towns with population less than 10.000). The population connection to the wastewater treatment plants increases (83 plants in total).

The collected wastewater is conducted to treatment plants or released directly into the rivers or sea without treatment. Wastewater treatment mitigates risks related to the public health, environmental pollution and endangerment of wildlife.

Generally, the wastewater treatment is carried out at the primary level using mechanical treatment, which removes minimum percentage of pollution. Share of mechanical treatment in overall wastewater treatment has considerably increased since the Zagreb Wastewater Treatment Plant become operational. 34 plants are equipped to operate at the secondary treatment level. Unfortunately, there is no  tertiary wastewater treatment yet. No progress has been made in development and use of artificial wetland for wastewater treatment, which is more cost-effective and environmentally sound method of wastewater treatment, particularly for small settlements.

Metadata Legal bases for the indicator: Environmental protection Act (OG 84/94) Water Act (OG 107/95) Regulation on Environmental Emission Inventory (OG36/96) Public Utility Management Act (OG 26/03) National water protection plan (OG 8/99) Regulations on limit values of Indices, hazardous and other substances in waste water (OG 40/99, 6/01, 14,01) Data source(s) : Annual reports (1995-2004) - Ministry of Environment Protection and Physical Planning Croatian Bureau of Statistics (data collection started in 1995) Geographical coverage: national Temporal coverage: 1995-2004 Methodology of data manipulation (measurements): Data on public sewage system are collected through regular annual reports of municipal business entities and municipal services which run the public water supply and the public sewage system in the settlements observed. Reports are partially based on records and documentation, and partially on estimations. Annual reports on public sewage system comprise all reporting units in the Register on Business Entities. Strength and weakness (at data level):